# PREVENTIVE MAINTENANCE

## Standard Operating Procedure

| Document ID: | SOP-MAINT-P100-2026 |
| --- | --- |
| Equipment: | Centrifugal Pump P-100 |
| Frequency: | Monthly / Quarterly / Annual |
| Revision: | 2.0 |
| Effective Date: | January 1, 2026 |

# 1. Purpose

This Standard Operating Procedure (SOP) establishes the preventive maintenance requirements for the P-100 series centrifugal pump to ensure optimal performance, minimize unplanned downtime, and extend equipment lifespan.

# 2. Scope

This SOP applies to all P-100 centrifugal pumps installed in the manufacturing facility, including units in production lines, utility systems, and backup installations.

# 3. Safety Requirements

Before performing any maintenance:

Apply Lockout/Tagout (LOTO) per SOP-SAFE-001

Wear required PPE: safety glasses, steel-toed boots, gloves

Ensure pump has cooled to ambient temperature

Verify zero energy state using test equipment

Have Material Safety Data Sheets (MSDS) available for fluids

# 4. Daily Inspection Checklist

Perform during operational rounds (estimated time: 5 minutes per pump)

Visual inspection for leaks: Check seal area, flange connections, drain plugs

Vibration check: Touch pump casing - should be smooth, no excessive vibration

Unusual noise: Listen for grinding, rattling, or cavitation sounds

Pressure gauges: Verify discharge pressure within 100-120 PSI range

Bearing temperature: Touch bearing housing - should not be too hot to touch

Motor sound: Listen for unusual motor noise or humming

# 5. Weekly Maintenance Tasks

Estimated time: 30 minutes

## 5.1 Lubricate bearings

Apply 2 pumps of NLGI Grade 2 grease to each bearing housing via grease fittings. Wipe excess grease.

## 5.2 Check coupling alignment

Visual inspection of motor-to-pump coupling. Look for signs of wear or misalignment (scoring, discoloration).

## 5.3 Inspect foundation bolts

Verify all mounting bolts are tight. Torque to 75 ft-lbs if loose.

## 5.4 Clean strainer (if equipped)

Remove and rinse suction strainer. Check for debris that might indicate upstream issues.

# 6. Monthly Maintenance Tasks

Estimated time: 1.5 hours

## 6.1 Mechanical Seal Inspection

With pump running, observe seal area for leakage

Acceptable: Slight misting or 1-2 drops per minute

Unacceptable: Continuous drip or stream

If leaking excessively, schedule seal replacement

## 6.2 Impeller Clearance Check

Impeller-to-casing clearance should be 0.015" to 0.030". Increased clearance reduces efficiency.

Shut down pump and apply LOTO

Remove casing drain plug

Insert feeler gauge through drain port

Measure clearance at top dead center

If clearance exceeds 0.040", adjust or replace impeller

## 6.3 Motor Current Draw Test

Normal current draw: 28-32 Amps at full load

Use clamp-on ammeter on each power lead

Record readings: L1: ___ A, L2: ___ A, L3: ___ A

Current imbalance should not exceed 10%

High current may indicate mechanical binding or voltage issues

Low current may indicate worn impeller or reduced flow

# 7. Quarterly Maintenance Tasks

Estimated time: 3-4 hours with parts

## 7.1 Mechanical Seal Replacement

Replace seal as preventive measure or when leakage exceeds limits.

Required Parts:

Mechanical seal assembly (P/N: P100-SEAL-MS01)

Seal chamber O-ring (P/N: P100-OR-125)

Shaft sleeve (inspect, replace if worn)

Procedure:

1. Isolate pump electrically and mechanically (LOTO)

2. Drain pump casing completely via drain valve

3. Disconnect coupling - remove coupling guard and loosen set screws

4. Remove seal housing bolts (8× M12, mark orientation)

5. Carefully extract seal housing assembly

6. Remove old seal - note direction of spring and faces

7. Clean seal chamber with lint-free cloth and solvent

8. Install new O-ring with light coating of seal lubricant

9. Install new seal per manufacturer instructions (carbon face toward product)

10. Reinstall seal housing - torque bolts to 45 ft-lbs in star pattern

11. Reconnect coupling with 0.005" gap

12. Fill pump casing with process fluid to prime

13. Remove LOTO, start pump, check for leaks

## 7.2 Vibration Analysis

Perform vibration analysis using handheld analyzer. Record readings in maintenance log.

| Measurement Point | Acceptable | Action Required |
| --- | --- | --- |
| Motor bearing (outboard) | < 0.3 in/sec | > 0.5 in/sec |
| Motor bearing (inboard) | < 0.3 in/sec | > 0.5 in/sec |
| Pump bearing (outboard) | < 0.4 in/sec | > 0.6 in/sec |
| Pump casing | < 0.5 in/sec | > 0.7 in/sec |

# 8. Annual Maintenance Tasks

Scheduled during plant shutdown. Estimated time: 8 hours with crane support

## 8.1 Complete disassembly and inspection

Remove pump from foundation. Disassemble completely. Inspect all wear parts.

## 8.2 Impeller replacement

Replace impeller if erosion, corrosion, or clearance exceeds limits. P/N: P100-IMP-SS316

## 8.3 Bearing replacement

Replace both radial ball bearings regardless of condition. P/N: P100-BRG-6308

## 8.4 Shaft inspection

Measure shaft at seal area with micrometer. Replace if worn > 0.005" from nominal.

## 8.5 Coupling replacement

Replace elastomer coupling element. P/N: P100-CPL-075

## 8.6 Motor megger test

Perform insulation resistance test. Should exceed 1 megohm. Report low readings.

# 9. Documentation Requirements

All maintenance activities must be recorded in:

• Computerized Maintenance Management System (CMMS)

• Equipment logbook located at pump station

• Include: date, time, technician name, work performed, parts used, findings

# 10. Spare Parts Inventory

Maintain minimum quantities on site:

| Part Number | Description | Min Qty |
| --- | --- | --- |
| P100-SEAL-MS01 | Mechanical seal assembly | 2 |
| P100-BRG-6308 | Radial ball bearing | 4 |
| P100-IMP-SS316 | SS316 impeller | 1 |
| P100-CPL-075 | Coupling elastomer | 2 |
| P100-OR-125 | Seal chamber O-ring | 5 |